## Experiment No. 5

## Title : Throughput and Latency Measurement Using Network Tools

## Aim : To measure and analyze network performance metrics (Throughput and Latency) using standard network tools via the command line interface (CLI), web-based simulators, and mobile interfaces over the college Wi-Fi network.

## Software & Hardware Requirements

- Hardware:

  - A Personal Computer / Laptop with Wi-Fi capability.

  - A Smartphone (Android or iOS) connected to the college Wi-Fi network.

- Software / Environment:

  - Windows Command Prompt (CMD) or macOS/Linux Terminal.

  - Any modern web browser (Google Chrome, Mozilla Firefox, Safari, Edge).

  - An online or local Python environment (Python 3.x).

  - Online testing suites: Speedtest by Ookla or Fast.com.

## Theory

Network performance analysis relies on two primary data transmission pillars to define service quality:

- Latency: This represents the time delay encountered by a data packet as it travels from the source device across the network to its destination and returns. Measured in milliseconds (ms), latency is functionally tested using a round-trip time (RTT) calculation. Low latency is critical for interactive applications such as voice calls or live streaming.

- Throughput: This is the practical volume of payload data successfully moved over the channel per unit of time. Measured in Megabits per second (Mbps), it represents operational speed and is segmented into:

  - Download Speed: The rate at which data is transferred from remote servers down to the client device.

  - Upload Speed: The rate at which the client device pushes data up to remote cloud environments.

## Experimental Setup & Procedure

### Part A: PC/Laptop via Command Prompt (CLI)

- Turn on your PC's Wi-Fi and connect to the College Wi-Fi network.

- Press Win + R, type cmd, and press Enter to open the Command Prompt (or open Terminal if on Mac/Linux).

- Type the following command to test latency to a stable external server (e.g., Google's public DNS): ping 8.8.8.8

- Observe the output packets sent, received, and the Round-Trip Time (RTT).

- Next, run a trace route to visualize the network hops your data takes : tracert 8.8.8.8

- (Note: Use traceroute 8.8.8.8 on Mac/Linux)

### Part B: Online Simulation & Visualization (Web Browser)

- Open your web browser.

- Navigate to an online HTML5 network simulator and test engine: Speedtest by Ookla or Fast.com.

- Click the "GO" button.

- Watch the real-time visual graph rendering your Throughput curve as it ramps up and stabilizes.

- Note down the recorded Ping, Download Throughput, and Upload Throughput.

### Part C: Mobile Device Setup

### If you want to quickly test your performance on a smartphone without loading third-party apps, use the built-in browser engine:

- Connect your smartphone to the College Wi-Fi.

- Open the default mobile browser (Safari or Chrome).

- Go to fast.com. This website is a minimalist, ad-free tool powered by Netflix that starts measuring download throughput automatically upon page load.

- Click "Show more info" once the test finishes to view your Latency (Unloaded vs. Loaded) and Upload Throughput.

## Python Code for Automation

This Python script uses the native OS environment to programmatically extract network latency.

import socket

import time

def measure_socket_latency(host="google.com", port=80, timeout=3):

"""

Estimates latency in an online sandbox by measuring TCP handshake time.

Uses Port 80 (HTTP) which online compilers keep open for basic outbound traffic.

"""

print(f"Connecting to {host}:{port} to estimate network latency...\n")

latencies = []

for i in range(4):

# Create a standard TCP Socket

sock = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

sock.settimeout(timeout)

start_time = time.time()

try:

# Connect to the host (initiates a 3-way handshake)

sock.connect((host, port))

end_time = time.time()

# Calculate elapsed time in milliseconds

latency = (end_time - start_time) * 1000

latencies.append(latency)

print(f"Reply from {host}: time={latency:.2f}ms")

except socket.error as e:

print(f"Connection attempt {i+1} failed: {e}")

finally:

# Cleanly close the socket connection

sock.close()

time.sleep(1) # Interval between requests

if latencies:

avg_latency = sum(latencies) / len(latencies)

print(f"\n--- Latency Statistics ---")

print(f"Minimum = {min(latencies):.2f}ms, Maximum = {max(latencies):.2f}ms, Average = {avg_latency:.2f}ms")

else:

print("\nCould not establish any socket connection. The online compiler environment has blocked all outbound traffic.")

if __name__ == "__main__":

# Using 'google.com' on standard Web Port 80

measure_socket_latency(host="google.com", port=80)

## Observations (Sample)

### Table 1: Command Line Interface (CLI) Results

| Target Host | Packets Sent | Packets Received | Packet Loss (%) | Minimum Latency (ms) | Maximum Latency (ms) | Average Latency (ms) |
| --- | --- | --- | --- | --- | --- | --- |
| 8.8.8.8 | 4 | 4 | 0% | e.g., 12ms | e.g., 18ms | e.g., 15ms |

### Table 2: Online Simulation Results (Browser vs. Mobile Browser)

| Platform | Download Throughput (Mbps) | Upload Throughput (Mbps) | Latency / Ping (ms) |
| --- | --- | --- | --- |
| PC Browser | e.g., 45.2 Mbps | e.g., 38.0 Mbps | e.g., 14 ms |
| Mobile Browser | e.g., 30.5 Mbps | e.g., 22.1 Mbps | e.g., 19 ms |

## Conclusion

In this experiment, the performance of the college Wi-Fi network was evaluated successfully using native OS utilities and browser-based real-time simulators. The data indicates how latency represents the responsiveness of the link, while download/upload metrics indicate the total payload capacity (throughput) available under current network loads. Differences in performance between the PC and mobile tests highlight how varying hardware interfaces interact with the same wireless infrastructure.