## Experiment No. 8

## Title : Analysis of Multipath Effects (Signal Fluctuation Observation)

Aim : To observe, analyze, and quantify the effects of multipath propagation (fading, constructive/destructive signal interference, and signal fluctuation) on wireless link quality using a Python-based channel path analyzer.

## Software & Hardware Requirements

- Hardware: Any PC or Laptop connected to the internet.

- Software: Any web browser to run Python online (e.g., OnlineGDB or Programiz).

## Theory

### Multipath Propagation : In a real-world wireless environment, an antenna transmits radio waves in all directions. Instead of traveling along a single straight path, signals bounce off surrounding objects (walls, buildings, vehicles, and terrain). As a result, multiple shifted versions of the same signal reach the receiving antenna via different paths:

- Direct Line-of-Sight (LoS) Path

- Reflected Paths (bouncing off walls/ground)

- Diffracted/Scattered Paths (bending around sharp edges)

### Signal Fluctuation & Fading Types

Because each path has a different distance, the waves arrive at the receiver with different time delays and phase angles:

- Constructive Interference: When path signals arrive in-phase, they add together, creating a signal peak (stronger RSSI).

- Destructive Interference: When path signals arrive out-of-phase (180circ difference), they cancel each other out, creating a deep fade (sharp drop in RSSI).

In mobile communication, this rapid variation over short distances or time intervals is called Small-Scale Fast Fading (modeled mathematically using Rayleigh or Rician distributions).

## Experimental Setup & Procedure

- Launch an online Python compiler in your web browser.

- Copy the Python code into the editor and execute it.

- Select an environment profile (e.g., Open Outdoor Field vs. Dense Urban Corridor).

- Observe how the superposition of multiple reflected signal paths causes rapid fluctuations in received signal strength (RSSI).

## Python Multipath Simulator Code

Python

import math

import random

import time

def generate_multipath_profile(environment_type):

"""

Simulates multipath reflection paths based on the chosen environment.

"""

# Environment configs: (Number of reflected paths, max reflection delay/attenuation)

profiles = {

"1": {"name": "Open Field (Rician / Strong LoS)", "paths": 2, "reflection_loss": 18.0},

"2": {"name": "Suburban Room (Moderate Multipath)", "paths": 5, "reflection_loss": 8.0},

"3": {"name": "Dense Urban Corridor (Rayleigh / High Multipath)", "paths": 10, "reflection_loss": 3.0}

}

return profiles.get(environment_type, profiles["2"])

def run_multipath_lab():

print("=============================================================")

print("     EXPERIMENT 8: MULTIPATH EFFECTS & SIGNAL FLUCTUATION LAB     ")

print("=============================================================\n")

print("Select Propagation Environment:")

print("  1. Open Field (Strong Direct LoS, minimal reflections)")

print("  2. Suburban Room (Moderate wall/furniture reflections)")

print("  3. Dense Urban Corridor (High multi-reflection environment)")

env_choice = input("\nEnter Choice (1-3): ")

env = generate_multipath_profile(env_choice)

print(f"\n[SYSTEM] Initializing Simulation: {env['name']}...")

print(f"[SYSTEM] Generating {env['paths']} active multipath signal components...\n")

time.sleep(0.5)

base_power = -50.0  # Base line-of-sight signal power in dBm

print("Time Step | LoS Signal | Reflection Phase | Net RSSI (dBm) | Fluctuation Status")

print("-" * 75)

rssi_history = []

# Run 15 consecutive temporal sampling intervals

for t in range(1, 16):

# Calculate phase variations across multiple reflection paths

total_real = 1.0  # Direct LoS path normalized vector

total_imag = 0.0

for path in range(env["paths"]):

# Random phase offset (0 to 2*pi) due to spatial path difference

phase = random.uniform(0, 2 * math.pi)

# Attenuation factor of reflected paths

attenuation = math.pow(10, -env["reflection_loss"] / 20.0) * random.uniform(0.5, 1.0)

total_real += attenuation * math.cos(phase)

total_imag += attenuation * math.sin(phase)

# Composite signal magnitude

composite_amplitude = math.sqrt(total_real**2 + total_imag**2)

# Convert magnitude multiplier to dB shift

fade_depth_db = 20 * math.log10(max(composite_amplitude, 0.01))

net_rssi = base_power + fade_depth_db

rssi_history.append(net_rssi)

# Categorize fluctuation state

if fade_depth_db > 2.0:

status = "🟢 Peak (Constructive)"

elif fade_depth_db < -6.0:

status = "🔴 Deep Fade (Destructive)"

else:

status = "🟡 Nominal Vector"

print(f"  t = {t:02d}s   |  -50.0 dBm  |   {math.degrees(phase):05.1f} deg    |   {net_rssi:05.1f} dBm   | {status}")

time.sleep(0.2)

# Statistical Summary

max_rssi = max(rssi_history)

min_rssi = min(rssi_history)

peak_to_fade = max_rssi - min_rssi

print("-" * 75)

print("\n--- MULTIPATH STATISTICAL SUMMARY ---")

print(f"  • Maximum Signal Level (Peak) : {max_rssi:.1f} dBm")

print(f"  • Minimum Signal Level (Fade) : {min_rssi:.1f} dBm")

print(f"  • Peak-to-Fade Fluctuation Span: {peak_to_fade:.1f} dB")

if peak_to_fade > 10.0:

print("  • ANALYSIS: High Multipath Distortion observed. Diversity/Equalization required.")

else:

print("  • ANALYSIS: Stable channel channel dynamics observed.")

print("=============================================================")

if __name__ == "__main__":

run_multipath_lab()

## Observations

### Table 1: Multipath Signal Fluctuation Data Log

Record the maximum, minimum, and fluctuation ranges across different environmental setups:

| Setup No. | Environment Type | Active Reflection Paths | Peak RSSI (dBm) | Deep Fade RSSI (dBm) | Total Fluctuation Span (dB) | Observed Multipath Effect |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Open Field | 2 |  |  |  |  |
| 2 | Suburban Room | 5 |  |  |  |  |
| 3 | Dense Urban Corridor | 10 |  |  |  |  |

## Results & Discussion

- Constructive Interference: When reflected signals arrived roughly in-phase with the direct signal, the combined power increased above the base level by up to +3 dB to +5dB.

- Destructive Interference: When reflection paths arrived out-of-phase, vector cancellation caused sharp signal drops (deep fades) of up to -15 dB or more.

- Environmental Impact: Dense environments with more reflection paths produce wider fluctuation spreads, requiring mitigation techniques such as Antenna Diversity, OFDMA, and Equalization.

## Conclusion

In this experiment, we analyzed the effects of multipath propagation on wireless communication. The data confirms that superposition of multiple reflected signal waves causes continuous signal strength fluctuations (fading). Dense environments with numerous reflecting obstacles experience significantly deeper signal fades compared to open line-of-sight paths.