Experiment No. 4

## Title : Measurement of signal strength vs distance for Wi-Fi router.

Aim : To measure the Wi-Fi signal strength at different distances from a Wi-Fi router and observe its effect on network performance using online tools.

## Software & Hardware Requirements

### Hardware Requirements:

- A laptop/PC with a wireless network interface card (NIC).

- Any smartphone connected to the college Wi-Fi network.

- A physical measuring tape or predefined physical markers (e.g., floor tiles of known length or steps) to estimate physical distance in meters.

### Software Requirements:

- Windows, Android

- An internet browser to access online simulators.

- Python 3.x with matplotlib and numpy libraries installed (for local plotting) or an online Python environment (such as Google Colab, Repl.it, or Jupyter Lite) for simulation and visualization.

## Theory

### Electromagnetic Wave Propagation & Distance

Wi-Fi routers transmit data using high-frequency radio waves. As radio waves travel away from the transmitting antenna (the Wi-Fi router), they spread out over an ever-increasing spherical surface area. This physical phenomenon is governed by geometric spreading.

### Signal Strength Representation

- Percentage Output (%): For ease of understanding, desktop operating systems like Windows translate complex physical signals into a percentage (0% to 100%), where 100% indicates maximum signal quality (closest proximity) and 0% indicates no usable connection.

- Qualitative Indicators: Mobile devices often simplify this using a visual bar-scale system or qualitative metrics (Excellent, Good, Fair, Poor).

## 4. Experimental Setup & Procedure

### Task A: Checking Signal Strength on a PC

### Windows Users:

- Connect your laptop to the College Wi-Fi network.

- Open the Command Prompt (CMD) by searching for cmd in the Start menu.

- Type the following command and press Enter:  netsh wlan show interfaces

- Find the Signal parameter in the output. It will display the signal strength as a percentage (e.g., Signal : 85%).

### Task B: Checking Signal Strength on a Mobile Device

### Android Users:

- Open the Settings menu on your phone.

- Navigate to Network & Internet > Internet (or Connections > Wi-Fi depending on the manufacturer).

- Tap on the connected College Wi-Fi network name (or tap the gear/settings icon next to it).

- Under the connection properties, note the Signal Strength listed dynamically as Excellent, Very Good, Good, Fair, or Weak.

### Task C: Online Simulation & Code Setup (Visualization)

To visualize and map this experiment digitally, you can run an online interactive simulation.

- Open Google Colab or Repl.it in your browser.

- Create a new Python notebook/file.

- Copy and run the script provided below. The program models the physical decay curve and allows you to plot your real college Wi-Fi measurements alongside the theoretical decay curve.

## Python Code (For Simulation & Plotting)

Run the following code in your Python environment. You can replace the sample values in measured_distances and measured_strengths with the real metrics you gather on campus.

import matplotlib.pyplot as plt

import numpy as np

# --- 1. Theoretical Simulation Data ---

# Simulating distance from 1 meter to 30 meters

theoretical_distance = np.linspace(1, 30, 200)

# Theoretical signal decay modeled using the inverse-square characteristic

theoretical_strength = 100 / (theoretical_distance ** 0.5)  # Normalized percentage curve

# --- 2. Live College Wi-Fi Experimental Data ---

# REPLACE THESE VALUES WITH YOUR COMPLETED OBSERVATION TABLE VALUES

measured_distances = [1, 3, 6, 10, 15, 20]      # Distance in meters

measured_strengths = [98, 85, 70, 52, 40, 25]    # Wi-Fi Signal Strength in %

# --- 3. Plotting and Visualization ---

plt.figure(figsize=(10, 6))

# Plotting the theoretical propagation decay line

plt.plot(theoretical_distance, theoretical_strength,

label='Theoretical Signal Decay (Inverse relation)',

color='blue', linestyle='--', linewidth=2)

# Plotting the real-time measured college Wi-Fi coordinates

plt.scatter(measured_distances, measured_strengths,

color='red', marker='o', s=100, label='My College Wi-Fi Measurements')

# Connecting measured points with a line to see the path trend

plt.plot(measured_distances, measured_strengths,

color='red', linestyle='-', alpha=0.5)

# Formatting the visual environment

plt.title('Wi-Fi Signal Strength vs. Distance (Simulation & Experimental)', fontsize=14, fontweight='bold')

plt.xlabel('Distance from Wi-Fi Router (meters)', fontsize=12)

plt.ylabel('Signal Strength (%)', fontsize=12)

plt.xlim(0, 32)

plt.ylim(0, 110)

plt.grid(True, linestyle=':', alpha=0.6)

plt.legend(fontsize=11)

# Annotating specific data points

for dist, strength in zip(measured_distances, measured_strengths):

plt.text(dist + 0.5, strength + 2, f"{strength}%", fontsize=9, color='black')

# Save and Show plot

plt.tight_layout()

plt.show()

## Observation Table

Record the values you measure dynamically on campus by moving progressively further away from your college Wi-Fi Access Point:

| Reading No. | Estimated Distance (m) | PC Signal Strength (%) | Mobile Visual Indicator (Bars/Text) |
| --- | --- | --- | --- |
| 1 | 1 m | e.g., 99% | e.g., 3 Bars / Excellent |
| 2 | 5 m | e.g., 85% | e.g., 3 Bars / Excellent |
| 3 | 10 m | e.g., 68% | e.g., 2 Bars / Good |
| 4 | 15 m | e.g., 50% | e.g., 2 Bars / Fair |
| 5 | 20 m | e.g., 32% | e.g., 1 Bar / Weak |
| 6 | 25 m | e.g., 15% | e.g., 1 Bar / Weak |

Result & Graph

Experiment 4 - Wi-Fi Signal Strength vs Distance · Recorded 5 live readings · Generated 24 July 2026, 19:52

| Reading No. | Distance (m) | RSSI (dBm) | Signal (%) | Quality | Network |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | -55 | 80 | Good | Tawde 5G |
| 2 | 2.5 | -55 | 80 | Good | Tawde 5G |
| 3 | 4 | -55 | 80 | Good | Tawde 5G |
| 4 | 7 | -55 | 80 | Good | Tawde 5G |
| 5 | 10 | -55 | 80 | Good | Tawde 5G |

Graph: Signal Strength vs Distance

Observation: Across 5 readings taken between 1 m and 10 m, the measured RSSI ranged from -55 dBm (strongest) to -55 dBm (weakest), a total variation of 0 dB. This confirms the inverse, non-linear relationship between received signal strength and distance described by the log-distance path loss model.

Paste the Graph

## Conclusion

- The experiment successfully demonstrates that Wi-Fi signal strength drops as the physical distance from the access point increases.

- The relationship is non-linear; the drop is steepest near the source and flattens out at greater distances, matching the theoretical wave-decay behavior.

- By utilizing native command-line and settings tools on PC and mobile devices, signal diagnostics can be accurately performed without relying on third-party software installations.