## Experiment No. 7

## Title : Study of Path Loss in an Indoor Environment (RSSI vs. Obstacles)

## Aim : To study and analyze the effect of physical indoor obstacles (e.g., Glass, Wood, Concrete, Metal) on wireless signal propagation and path loss by calculating RSSI degradation using an interactive Python-based indoor channel simulator.

## Software & Hardware Requirements

- Hardware: Any PC or Laptop connected to the internet.

- Software: Any web browser to run Python online (e.g., OnlineGDB or Programiz).

## Theory

### Indoor Path Loss & Obstacle Attenuation

In an indoor wireless environment, signal strength degrades due to two main factors:

- Free-Space Path Loss (FSPL): The natural spreading of electromagnetic waves over distance.

- Material Absorption Loss: Reduction in power caused by radio waves penetrating physical barriers.

The received signal power (RSSI) in dBm can be modeled using the Log-Distance Path Loss Model with obstacle attenuation factors:

RSSI(d) = Ptx + Gtx + Grx - (FSPL(d0) + 10 × n × log10(d/d0)) - ΣLobstacle

### Where:

- RSSI(d) = Received Signal Strength Indicator at distance d (dBm)

- P_tx = Transmitted power (dBm)

- G_tx = Gain of the transmitting antenna (dBi)

- G_rx = Gain of the receiving antenna (dBi)

- FSPL(d0) = Free Space Path Loss at the reference distance d0 (dB)

- n = Path loss exponent (depends on the environment)

  - Free space: 2

  - Indoor office: 2–4

  - Urban area: 2.7–5

- d = Distance between transmitter and receiver (m)

- d0 = Reference distance (usually 1 m)

- Σ L_obstacle = Total attenuation due to obstacles (walls, doors, furniture, glass, etc.) in dB

### Typical Material Attenuation Values (2.4 GHz Band):

- Clear Glass Window: 2 dB

- Wooden Door: 4 dB to 6 dB

- Concrete / Brick Wall: 12 dB to 18 dB

- Reinforced Metal / Elevator Core: 25 dB to 30 dB

## Experimental Setup & Procedure

### Part A: Physical Field Verification (Classroom Activity)

- Turn on the Wi-Fi Hotspot on a smartphone (Transmitter) and place it at a fixed point inside a room.

- Open a Wi-Fi analyzer mobile app (e.g., NetSpot or Wi-Fi Analyzer) on a second phone (Receiver).

- Record RSSI readings (dBm) at 2 meters, 5 meters, and 10 meters in direct line-of-sight.

- Move behind a wooden door, glass partition, and concrete wall at the same distances and log the drop in signal power.

### Part B: Python Indoor Path Loss Simulator

- Open an online Python compiler in your browser.

- Copy the Python script given below into the editor and execute it.

- Select different obstacle materials and distances to generate real-time RSSI readings, signal bar meters, and ASCII propagation graphs.

## Python Indoor Path Loss Simulator

import math

import random

import time

def render_signal_meter(rssi_value):

"""Generates an ASCII signal strength bar graph."""

# Map RSSI from -110 dBm (worst) to -30 dBm (best) onto 25 characters

scale = int((rssi_value - (-110)) / 80 * 25)

scale = max(0, min(25, scale))

bar = "█" * scale + "░" * (25 - scale)

return f"[-110 dBm] |{bar}| [-30 dBm]"

def run_indoor_path_loss_lab():

print("==============================================================")

print("      EXPERIMENT 10: INDOOR PATH LOSS & OBSTACLE SIMULATOR        ")

print("============================================================\n")

time.sleep(0.3)

# OBSTACLE DATABASE

obstacles = {

"1": {"name": "Free Space (Direct Line of Sight)", "loss": 0, "icon": "CLEAR"},

"2": {"name": "Clear Glass Window Partition", "loss": 2, "icon": "GLASS"},

"3": {"name": "Solid Wooden Door", "loss": 5, "icon": "WOOD"},

"4": {"name": "Concrete / Brick Wall", "loss": 14, "icon": "BRICK"},

"5": {"name": "Reinforced Metal Shield", "loss": 28, "icon": "METAL"}

}

print("Select an Indoor Obstacle Scenario:")

for key, val in obstacles.items():

print(f"  {key}. {val['name']} (-{val['loss']} dB)")

choice = input("\nEnter Choice (1-5): ")

selected_obstacle = obstacles.get(choice, obstacles["1"])

try:

distance = float(input("Enter distance in meters (1 to 20m): "))

except ValueError:

distance = 3.0

print("Invalid entry. Defaulting distance to 3.0 meters.")

distance = max(1.0, min(20.0, distance))

print("\n[CALCULATING] Computing Indoor Signal Attenuation Profile...")

time.sleep(0.5)

# LINK BUDGET CALCULATION PARAMETERS

p_tx = 15.0  # Transmit Power in dBm (Typical Wi-Fi Router)

freq_mhz = 2400.0  # 2.4 GHz

path_loss_exponent = 3.0  # Indoor obstructed environment exponent

# Free Space Path Loss at 1m reference distance (d0)

fspl_1m = 20 * math.log10(1) + 20 * math.log10(freq_mhz) - 27.55

# Distance Path Loss

distance_loss = 10 * path_loss_exponent * math.log10(distance)

# Total Attenuation

obstacle_loss = selected_obstacle["loss"]

fading_shadowing = random.uniform(-1.5, 1.5)  # Multipath flutter

total_path_loss = fspl_1m + distance_loss + obstacle_loss + fading_shadowing

rssi = p_tx - total_path_loss

# OUTPUT RESULTS

print("\n--- VISUAL PROPAGATION PATH ---")

pad = "┈" * int(distance)

print(f"📡 [TX ROUTER] {pad} 🛑 [{selected_obstacle['icon']}] {pad} 📱 [RX PHONE]")

print("=" * 66)

print("\n--- LINK METRICS BREAKDOWN ---")

print(f"  • Base Transmit Power (P_tx)  : +{p_tx:.1f} dBm")

print(f"  • Distance Path Loss ({distance}m) : -{fspl_1m + distance_loss:.1f} dB")

print(f"  • Material Attenuation Loss  : -{obstacle_loss:.1f} dB")

print(f"  • Computed Signal (RSSI)     : {rssi:.1f} dBm")

print(f"\nSignal Meter: {render_signal_meter(rssi)}")

# HEALTH DIAGNOSIS

print("\n--- CONNECTION VERDICT ---")

if rssi >= -65:

print("  STATUS: EXCELLENT (Full speed, ultra-low packet loss)")

elif rssi >= -80:

print("  STATUS: GOOD (Stable connection, minor throughput drop)")

elif rssi >= -92:

print("  STATUS: POOR (High latency, web pages load slowly)")

else:

print("  STATUS: OUT OF RANGE (Connection dropped due to obstacle)")

print("=============================================================\=")

if __name__ == "__main__":

run_indoor_path_loss_lab()

## Observations

### Table 1: Indoor Obstacle Attenuation Data Log

Run the simulation or field test across different obstacles and distances to fill in the observation table:

| Test No. | Obstacle Material | Distance (d) | Base Path Loss (dB) | Material Loss (dB) | Calculated RSSI (dBm) | Connection Status |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Line of Sight (Free Space) | 3 m |  |  |  |  |
| 2 | Clear Glass Window | 3 m |  |  |  |  |
| 3 | Solid Wooden Door | 5 m |  |  |  |  |
| 4 | Concrete / Brick Wall | 5 m |  |  |  |  |
| 5 | Reinforced Metal Shield | 10 m |  |  |  |  |

## Results & Discussion

- Impact of Distance: As distance increases, RSSI drops logarithmically according to the path loss exponent (n is approx 3).

- Impact of Obstacle Density: Low-density materials like Glass (2 dB) cause minimal signal degradation, whereas high-density materials like Concrete (14 dB) and Metal (28 dB) absorb and reflect radio waves heavily, leading to dead zones.

## Conclusion

In this experiment, we studied the effect of distance and physical indoor barriers on RSSI degradation. The results confirm that physical barriers add significant attenuation to the total path loss, proving that indoor network planning requires strategic Access Point placement to minimize penetration through dense materials like concrete and metal.