## Experiment No. 6

## Title :  Study of Bluetooth Communication (Pairing, Discovery, and Range Testing)

## Aim : To study the concepts and behavior of Bluetooth Communication using mobile devices by:

## Understanding the Discovery, Inquiry, and Paging phases of short-range wireless links.

## Observing and documenting the Secure Simple Pairing (SSP) numeric comparison authentication mechanism.

## Conducting a structured Range and Path Loss Field Test to understand signal degradation over distance and boundary limits.

## Simulating and visualizing Bluetooth network protocol mechanics through an interactive terminal application.

## Software & Hardware Requirements

### Hardware Requirements:

## Primary Device (Master): A Laptop or PC with an active built-in Bluetooth adapter.

## Secondary Device (Slave): A Smartphone (Android or iOS) with Bluetooth capability.

## Measurement Tool: A measuring tape (or estimated physical pacing steps) to mark distances (1m, 5m, 10m, 15m, 20m).

### Software Requirements:

## Operating System: Windows, macOS, or Linux with native Bluetooth GUI.

## Execution Environment: Any local or web-based Python 3.x interpreter (e.g., OnlineGDB, Programiz, Replit, or local IDLE).

## Theory

### Basic Bluetooth Concepts

## Bluetooth is a short-range, low-power, wireless communication technology designed for ad-hoc voice and data transmission. It operates in the globally unlicensed 2.4 GHz ISM (Industrial, Scientific, and Medical) band (2400 - 2483.5 MHz).

## To avoid interference from other radio signals sharing the same band (such as college Wi-Fi or microwave ovens), Bluetooth employs Frequency-Hopping Spread Spectrum (FHSS). It divides the band into 79 RF channels (spaced 1 MHz apart) and hops pseudo-randomly across them 1,600 times per second.

### Bluetooth Protocol Phases

## Inquiry (Discovery): A master device scans the physical vicinity by broadcasting inquiry messages. Nearby discoverable devices respond with their unique 48-bit Bluetooth Device Address (BD_ADDR) and local user-friendly device name.

## Paging (Connection Setup): The master target-signals a specific BD_ADDR to establish a synchronized physical hop-sequence connection channel.

## Secure Simple Pairing (SSP): SSP handles security by verifying identity through a Numeric Comparison method. Both devices generate and display a matching temporary 6-digit passcode. Confirming the passcode validates the link, exchanges a cryptographic Link Key, and establishes a secure bond (pairing).

## Range & Attenuation (Path Loss): Transmitters are categorized into classes. Most modern smartphones use Class 2 transmitters with a nominal operating range of approximately 10 meters (33 feet). As physical separation increases, the signal experiences path loss (attenuation), leading to higher bit-error rates, data buffer stutters, and eventual complete link disconnection.

## Experimental Setup & Procedure

### Part A: Verifying Local Host Bluetooth via Terminal

## To check if the underlying Bluetooth controller hardware is responsive:

## On Windows (PowerShell): Open PowerShell and execute:

## PowerShell : Get-PnpDevice -Class Bluetooth | Select-Object FriendlyName, Status

### Part B: Physical Mobile Device Pairing & Service Mapping

## On your smartphone, open Settings > Bluetooth and switch it on. Make sure your device is set to "Visible to all nearby devices" (Discoverable mode).

## On your Laptop, navigate to Bluetooth Settings and click "Add Device" (or scan).

## Identify your mobile phone in the list of discovered devices. Click on it to initiate pairing.

## Observe the matching 6-digit numeric comparison code pop up on both screens. Check "Yes / Pair" on both devices to verify authentication and finalize bonding.

## Service Mapping: Locate your paired smartphone's connection properties on your PC. Note which Bluetooth service profiles (such as A2DP Audio, Audio Gateway, OBEX File Transfer, or HID Keyboard/Mouse) are actively exposed by your phone.

### Part C: Range and Link Quality Performance Challenge

## Set up a straight-line path in an open hallway or lab. Mark distance increments at 1m, 5m, 10m, 15m, and 20m using estimated steps (approx. 1 step approx 0.75m).

## Leave the laptop on a table. Pair it with your smartphone and start a continuous Bluetooth stream (such as a large file transfer, a ping utility, or real-time audio playback to wireless earbuds).

## Walk away slowly with your mobile device and pause at each marked distance.

## Record the connection's behavior at each step (e.g., instant transfer, noticeable delay, intermittent audio stuttering, or complete signal drop).

## Python Interactive Protocol Simulator

## This code simulates the physical Bluetooth protocols, pairing states, and distance attenuation logs. It runs without requiring local physical Bluetooth hardware.

## import time

## import random

## def run_bluetooth_simulator():

## print("=================================================================")

## print("      BLUETOOTH PAIRING, DISCOVERY, & RANGE SIMULATION          ")

## print("=================================================================\n")

## time.sleep(1)

## # 1. DISCOVERY STAGE

## print("[PHASE 1] Initializing 2.4 GHz RF Inquiry Scan...")

## time.sleep(1.2)

## nearby_devices = [

## {"id": 1, "name": "Lab_Smartphone_A", "mac": "9C:D6:F7:11:22:AA", "class": "Class 2 (Mobile)"},

## {"id": 2, "name": "Student_Headset", "mac": "00:1A:7D:DA:71:BB", "class": "Class 2 (Audio)"},

## {"id": 3, "name": "College_IoT_Printer", "mac": "AA:BB:CC:11:22:CC", "class": "Class 1 (Industrial)"}

## ]

## print("Discovered Active Bluetooth Devices in Piconet Range:")

## for dev in nearby_devices:

## print(f" -> Device Name: {dev['name']:<22} | MAC: {dev['mac']:<20} | {dev['class']}")

## time.sleep(0.6)

## print("\nSelecting 'Lab_Smartphone_A' to establish paging connection...")

## time.sleep(1)

## # 2. PAIRING STAGE (Secure Simple Pairing)

## print("\n[PHASE 2] Initiating Secure Simple Pairing (SSP) handshake...")

## time.sleep(1.2)

## # Generate 6-digit pin code

## ssp_pin = random.randint(100000, 999999)

## print("-----------------------------------------------------------------")

## print(f"  [PAIRING CONFIRMATION REQUEST]")

## print(f"  Passkey Displayed on Host: {ssp_pin}")

## print(f"  Passkey Displayed on Phone: {ssp_pin}")

## print("-----------------------------------------------------------------")

## input("\n[ACTION Required] Press ENTER to confirm that the passkeys match...")

## print("STATUS: Keys matched! Secure cryptographic link key stored. Devices Bonded.\n")

## time.sleep(1)

## # 3. INTERACTIVE PROPAGATION & RANGE TESTING CHALLENGE

## print("[PHASE 3] Interactive Link Quality & Range Degradation Test")

## print("Enter your custom range parameters to evaluate performance limits:")

## try:

## dist = float(input("Enter simulated distance in meters (1 to 25): "))

## except ValueError:

## dist = 5.0

## print("Invalid entry. Defaulting to 5.0 meters.")

## print("\nSelect your current testing environment:")

## print("1. Line of Sight (LoS) - Clear open hallway")

## print("2. Non-Line of Sight (NLoS) - Obstructed walls / High 2.4GHz interference")

## env_choice = input("Enter choice (1 or 2): ")

## attenuation_multiplier = 4.5 if env_choice == "2" else 2.5

## print("\nSending simulated packet blocks to measure performance...")

## print("-" * 65)

## print(f"{'Packet Block':<15}{'Estimated Signal Strength':<28}{'Status'}")

## print("-" * 65)

## loss_count = 0

## for block_id in range(1, 6):

## # Calculate simulated signal strength (RSSI) with random fluctuations

## base_rssi = -30 - (dist * attenuation_multiplier) + random.uniform(-4, 4)

## # Determine status based on signal levels

## if base_rssi > -65:

## status = "SUCCESS (Optimal Connection)"

## elif base_rssi > -85:

## status = "SUCCESS (High Latency / Intermittent)"

## elif base_rssi > -98:

## status = "STUTTERING (Packet Loss Detected)"

## loss_count += 1

## else:

## status = "LOST (Connection Timeout)"

## loss_count += 1

## print(f"Block #{block_id:<8}  {base_rssi:.1f} dBm{'':<15} {status}")

## time.sleep(0.8)

## success_rate = ((5 - loss_count) / 5) * 100

## print("-" * 65)

## print(f"EXPERIMENT METRIC ANALYSIS:")

## print(f"  * Total Successful Delivery Ratio: {success_rate}%")

## print(f"  * Measured Distance Boundary Level: {dist} meters")

## if success_rate >= 80:

## print("  * System Diagnosis: Stable physical channel connection.")

## elif success_rate >= 40:

## print("  * System Diagnosis: Severely degraded link. Frame drop and buffer stutters.")

## else:

## print("  * System Diagnosis: Link Loss. Out of range or blocked by obstacles.")

## print("=================================================================")

## if __name__ == "__main__":

## run_bluetooth_simulator();

## Observations

### Table 1: Bluetooth Discovery & Service Profiling

| ## Discovered Name | ## MAC Address (BD_ADDR) | ## Class / Category | ## Supported Profile Services |
| --- | --- | --- | --- |
| ## e.g., Galaxy S24 | ## e.g., 9C:D6:F7:11:22:AA | ## Class 2 | ## A2DP (Audio Source), OBEX (File Transfer) |

### Table 2: Signal Quality vs. Distance Field Test (Open Area)

| ## Paced Distance (m) | ## Link Connectivity State | ## Practical Data/Stream Performance |
| --- | --- | --- |
| ## 1 m | ## Connected | ## Flawless connection. Instantaneous file/audio transfers. |
| ## 5 m | ## Connected | ## Very stable. No buffering, stuttering, or delay. |
| ## 10 m | ## Weak Connection | ## Sluggish response times. File transfers take longer to start. |
| ## 15 m | ## Intermittent | ## Audio/data stream stutters, drops, and tries to reconnect. |
| ## 20 m | ## Disconnected | ## Pairing link lost entirely due to signal path loss. |

## Conclusion

## Through this practical, we successfully studied the fundamentals of Bluetooth short-range networks (piconets).

## The experimental stages confirmed that Secure Simple Pairing (SSP) provides standard identity authentication, while the range-testing segment demonstrated how physical distance and environmental factors cause signal attenuation. The tests confirm that standard Class 2 Bluetooth transmitters are highly stable within 5 - 10 meters, beyond which path loss leads to connection timeouts and link failure.